Clustering Analysis of Fast Ion Driven Instabilities

John Gresl1
William Heidbrink1
Shaun Haskey2
Boyd Blackwell3

1Department of Physics and Astronomy, University of California Irvine, California 92612, USA
2Princeton Plasma Physics Lab, Princeton University, New Jersey 08540, USA
3Plasma Research Laboratory, Research School of Physics and Engineering, The Australian National University, Canberra, ACT 0200, Australia

Submitted: xxxxx

Abstract

Beam ions often drive Alfvén eigenmodes and other instabilities unstable in DIII-D. Many of these modes have been unambiguously identified but some frequently occurring features have been neglected. In this work, datamining and analysis techniques1 that successfully analyzed magnetics data from the H-1NF Heliac are applied to arrays of magnetic and electron cyclotron emission (ECE) data from DIII-D. The clustering techniques group instabilities with similar toroidal magnetic features into clusters of identical mode numbers. Similar analysis is performed on DIII-D’s poloidal magnetic array and ECE probes. Something about ECE results here, later.

Introduction

Plasma is one of the four fundamental states of matter and makes up an estimated 99% of the matter in the observable universe. A plasma can be thought of as an quasineutral medium of unbound positively and negatively charged particles. These moving charged particles generate local magnetic fields which affect the motion of other nearby particles leading to collective behavior, or motion that depends on the physical state of the plasma in local regions.

Harnessing nuclear fusion is one of the prime motivators of studying plasma physics. Nuclear fusion produces more energy per amount of fuel than any other available fuel source. For instance, one gallon of heavy water (water with all of the hydrogen atoms replaced with deuterium atoms) provides 10,000 times more energy when fused than a gallon of gasoline. Deuterium-tritium (D-T) plasmas are known as the most efficient plasma for energy production due to their high mass-to-charge ratio, making it easier to overcome the weak force and fuse together. There are four main reactions that occur in D-T plasmas.

D+D→0.82 MeV  3He+2.45 MeV n

D+D→1.01 MeV T+3.02 MeV p

D+ 3He→3.71 MeV α+14.64 MeV p

D+T→3.56 MeV α+14.03 MeV n

Above, D is a deuterium ion, T is a tritium ion, p is a proton, n is a neutron, and α is a  2He nucleus.

Many research plasmas require temperatures on the order of 108 K, making the plasma hot enough to destroy anything it comes into contact with2. For this reason, strong magnetic fields are used to shape and contain the plasma. The introduction of these magnetic fields, while necessary, can lead to some undesirable effects such as unwanted resonances, instabilities, and particles escaping from the plasma and colliding with the inner walls. The study of these interactions is crucial for creating higher quality magnetically confined plasmas.

Background Physics

Cyclotron Motion

Charged particles trapped in magnetic fields exhibit circular orbits and is known as cyclotron motion. For a non-stationary particle moving at velocity, v, with charge, q, in a magnetic field, B, the force, F is equal to:

FB=q(v × B)

It is easy to show that there is no work done by this force.

dWdt=F ∙v= qv × B∙v=0

Since v × B is perpendicular to both v and B, where W is work. The total energy of the particle does not change. We can rewrite velocity in terms of a new basis

v=v⊥+ v∥

where v⊥ and v∥ represent the components of velocity perpendicular and parallel to the magnetic field, respectively. We can then rewrite FB.

FB=qv⊥+v∥× B

FB=q[v⊥× B+v∥× B]

FB=q[v⊥× B]

We can say v∥×B goes to zero since its magnitude, v∥×B, is equal to 0 since v∥ is parallel to B by definition.

This force causes charged particles to rotate in circles as they travel along magnetic field lines, all while keeping their parallel velocity constant (see figure 1). Equating the magnetic force to the centripetal force and solving for the radius of curvature yields an expression for the radius of curvature of this gyrating charge known as the Larmor radius, rL.

qv⊥B=mv⊥2rL→rL=m v⊥qB

B

Figure 1. Graphical representation of the path of a charged particle (blue) travelling along a magnetic field line (yellow).

From the Larmor radius, one can also determine a parameter known as the cyclotron frequency, Ωc:

Ωc=2πf=v⊥rL=qBm

When electrons undergo cyclotron motion, they emit radiation in the form of photons. This is known as electron cyclotron emission (ECE) and is an important diagnostic when studying plasmas.

Figure 2. Geometry of an Alfvén wave. The wave has k along B0, E1 and j1 perpendicular to B0 and B1 and v1 perpendicular to both B0 and E1.

Alfvén Waves and Eigenmodes

Alfvén waves are low-frequency (relative to Ωc) travelling oscillation that travels along a magnetic field line3. The motion of an Alfvén wave is analogous to a wave travelling along a stretched string. In this analogy, the tension in the magnetic field lines is the same as the tension in the string, and the Alfvén wave is the result of the “plucking” of the string.

From Maxwell’s equations, we can derive the velocity at which these Alfvén waves travel to be va≡B0/μ0  ρ. [See appendix A for actual derivation]. The Alfvén velocity refers to the characteristic speed in which perturbations of the lines of the force travel.

Particles within the plasma may interact undesirably with the external magnetic fields creating modes or local instabilities. Many of these instabilities are detrimental to the plasma’s health and can sometimes cause energetic particles to escape the plasma and damage expensive equipment on the inner walls. For our experiment, we are interested in the particles that become trapped, or locked, in the Alfvén waves and refer to them as Alfvén eigenmodes (AE’s).

The Device and Diagnostics

This experiment is done exclusively with data gathered from the DIII-D tokamak and diagnostics which have already been installed. DIII-D is a torus-shaped plasma device which confines plasma by using strong magnetic fields. The tokamak has a major radius of 1.67 m and a minor radius of 0.67 m. A coil of large electromagnets (B-coils) are positioned like rings around the tokamak to

Figure 3. Cartoon of top down view of DIII-D. The 4-pairs of neutral beam injectors are shown in purple and the various toroidal magnetic Mirnov probes are shown in green with their angular position indicated.

produce toroidal magnetic fields on the order of 2-3 Tesla. Field shaping coils are also used to provide additional stability and confinement to the plasma.

DIII-D also utilizes 4 pairs of neutral beam injectors which function as an auxiliary energy source for the plasma. These neutral beams supply high energy neutral particles to the plasma where they can interact with other particles by collisions. The orientation of these neutral beam injectors can be seen in figure 4. One capability which sets the neutral beams at DIII-D apart from others is its ability to run their beams at a very high confidence over the range 40 kV to 75 kV without the need for special calorimetry or test shots4.

Magnetic Mirnov Probes

Among the many diagnostics at DIII-D, the magnetic Mirnov probes are one of the most important. The probes consist of coils of wire which measure the changing magnetic field dB/dt caused by moving charges in the plasma. We are interested in the main toroidal magnetic Mirnov array and the 322-degree poloidal magnetic Mirnov array. We will refer to these arrays as the toroidal array and the poloidal array, respectively. The toroidal array consists of 14 probes and lies on the toroidal midplane of the tokamak. The arrangement of the 14 probes in the toroidal array can be seen in figure 3, along with the orientation of the neutral beam injectors. The poloidal array consists of 31 (verify) probes and encircles the tokamak poloidally at the 322-degree mark.

Electron Cyclotron Emission (ECE)

The ECE array at DIII-D consists of a 40-channel heterodyne radiometer that provides the electron temperature at a given radius and time. Fluctuations within the plasma can cause the cyclotron frequencies of the electrons, Ωc, to vary in time and space. Alfvén eigenmodes frequently affect the mode structure and can cause fluctuations in Ωc which make the ECE arrays a helpful tool for studying Alfvén eigenmodes.

Data Analysis Techniques

Clustering Analysis

Clustering is the act of sorting objects into groups based on user determined clustering parameters. Performing clustering analysis on bulky data sets is an effective method of uncovering an underlying structure. One common clustering algorithm, known as k-means uses distance between two points as the clustering parameter and groups objects together based on how far apart they are. A general k-means algorithm behaves as follows: let’s say you have n-data points in a 2-dimensional space that you would like to separate into k-clusters. The algorithm begins by selecting k-cluster centers (or means) and then grouping each data point to the mean that it is closest too. Once each data point has been classified, the mean of each group is calculated and all the data points are regrouped based on this new set of means. This process of grouping, calculating the mean, and regrouping will iterate a set amount of times, or until a convergence criteria is reached. A more in-depth example of this process is given in Appendix B using the python programming language.

Bibliography

1.	Haskey, S. R., Blackwell, B. D. & Pretty, D. G. Clustering of periodic multichannel timeseries data with application to plasma fluctuations. Comput. Phys. Commun. 185, 1669–1680 (2014).

2.	Heidbrink, W. W. Basic physics of Alfvn instabilities driven by energetic particles in toroidally confined plasmas. Phys. Plasmas 15, (2008).

3.	Chen, F. F. Introduction to Plasma Physics and Controlled Fusion. (Springer, 1984).

4.	DIII-D Beams. (2016). Available at: https://diii-d.gat.com/diii-d/Beams_feat. (Accessed: 16th May 2017)

Appendix

A. Derivation of Alfvén Velocity

Refer to figure 2 for definitions of variables and terms. From Maxwell’s equations, we have

|  | ∇×∇×E1=-kk ∙E1+k2E1=ω2c2E1+i ωϵ0 c2j1 | [1] |
| --- | --- | --- |

Since k is only oriented in the z direction and E1 is only oriented in the x direction, only the x component if nontrivial. Our equation becomes

|  | ϵ0ω2-c2k2E1=-iωn0e(vix-vex) | [2] |
| --- | --- | --- |

The thermal motion of the particles is not important, so we can use the solution to the ion equation of motion, [3], with Ti equal to zero.

|  | M∂vi1∂t=-e∇ϕ1+evi1×B0 | [3] |
| --- | --- | --- |

Which separates into

|  | -iωMvix=-eE1+eviyB0 -iωMviy= -evixB0 | [4] [5] |
| --- | --- | --- |

Solving for viy explicitly in [5] yields

|  | viy=eB0iωMvix=Ωciωvix | [6] |
| --- | --- | --- |

We can plug this into [4] and solve for vix

|  | vix=-1iωM-eE1+eΩciωvix=eiMω1-Ωc2ω2-1E1 | [7] |
| --- | --- | --- |

Which is the velocity of the ions in the x direction. To find the electron velocity, we consider the transformation where M→me,   e→-e,   Ωc→ωc and the limit where ωc2≫ω2

|  | vex=eimeωω2ωc2E1→0 | [8] |
| --- | --- | --- |

This term goes to zero since the ωc2≫ω2; the Larmor gyrations are negligible so the electrons only have an E×B drift in the y direction. Substituting [7] and [8] into [2] yields

|  | ϵ0ω2-c2k2E1=-iωn0eeiMω1-Ωc2ω2-1E1 ω2-c2k2=Ωp21-Ωc2ω2-1 | [9] [10] |
| --- | --- | --- |

where Ωp2=n0e2/ϵ0M is the square of the ion plasma frequency.

Now, we take into consideration the fact that Alfvén wave frequencies are much below the ion cyclotron frequency, ω2≪Ωc2. In this limit, [10] becomes

|  | ω2-c2k2=-Ωp2ω2Ωc2=-n0e2ϵ0Mω2M2e2B02=-ω2ρϵ0B02 | [11] |
| --- | --- | --- |

|  | ω2k2=c21+ρϵ0B02 | [12] |
| --- | --- | --- |

where ρ=n0M is the mass density. Recall that ϵ0=μ0c2-1 and [12] becomes

|  | ω2k2=c21+ρμ0B02c2 | [13] |
| --- | --- | --- |

The denominator of [13] can be recognized as the relative dielectric constant for low frequency perpendicular motions, ϵR.

|  | ωk=cϵR | [14] |
| --- | --- | --- |

ϵR is much larger than one for most laboratory plasmas3, so [13] can be approximated as

|  | ω2k2=c2B02 ϵ0ρ=B02μ0ρ | [15] |
| --- | --- | --- |

And we have arrived at our result for the Alfvén velocity.

|  | va≡ωk=B0μ0ρ | [16] |
| --- | --- | --- |

B. K-Means Clustering Example in Python

This code example may be found in my GitHub repository at:
https://github.com/SyntaxVoid/PyFusionDIIID/blob/master/Writing/ClusteringExample.py

import matplotlib.pyplot as plt
import numpy as np
import random


## 0. Create a Point class and some usefull functions
class Point:
    ## A point is created by calling Point(x,y)
    def __init__(self, x, y):
        self.x = x
        self.y = y
        return

    def plot_self(self, axes, marker, color):
        ## Plots itself on a given matplotlib axes with a certain color
        axes.plot(self.x, self.y, marker=marker, color=color)


def distance(point1, point2):
    ## Returns the distance between point1 and point2
    return np.sqrt((point2.x - point1.x) ** 2 + (point2.y - point1.y) ** 2)


def random_point(a, b):
    ## Returns a point object with x,y values between a and b.
    return Point(random.uniform(a, b), random.uniform(a, b))


def cluster_mean(cluster):
    ## Returns the mean value of a cluster of points
    x_sum = 0
    y_sum = 0
    for point in cluster:
        x_sum += point.x  ## a += b represents a = a + b
        y_sum += point.y
    x_avg = x_sum / len(cluster)
    y_avg = y_sum / len(cluster)
    return Point(x_avg, y_avg)


## 1. Create a random set of 1000 data points
n_points = 1000
points = []
for i in range(n_points):
    points.append(
        random_point(2, 8))  # This will add a random point with x,y values between 2 and 8 to our array of points

## 2. Generate our first set of 7 random cluster centers
n_clusters = 7
means = []
for cluster_id in range(n_clusters):
    means.append(random_point(2, 8))

## 3. Perform clustering
n_iterations = 50
for i in range(n_iterations):  ## Iterate the process 50 times
    clusters = []  ## Clear our clusters before each iteration
    for cluster_id in range(n_clusters):
        clusters.append([])  ## Create an empty cluster for as many clusters as we have
    for point in points:  ## Iterate over every data point
        dists = []
        for mean in means:
            dists.append(distance(point, mean))  ## Calculates the distance from the current point to each mean
        closest_mean_index = dists.index(min(dists))  ## Returns the index of the smallest distance
        clusters[closest_mean_index].append(point)  ## Add the point to the appropriate cluster
    for cluster_id in range(n_clusters):
        means[cluster_id] = cluster_mean(clusters[cluster_id])  ## Calculate new mean for each cluster

## 4. Plot your clusters
colors = ["blue", "orange", "red", "green", "black", "yellow", "cyan"] ## Add more colors here for each cluster you have
figure, axes = plt.subplots(1,1) ## Create our figure and axes objects
for cluster_id in range(n_clusters): ## Iterate over each cluster
    for point in clusters[cluster_id]: ## Iterate over each point in the cluster
        point.plot_self(axes=axes, marker="o", color=colors[cluster_id])
    mean = means[cluster_id]  ## Next few lines plot circles around each cluster
    mean_radius = max([distance(point, mean) for point in clusters[cluster_id]])
    axes.add_artist(plt.Circle((mean.x, mean.y), mean_radius, color=colors[cluster_id], alpha=0.25))

plt.xlabel("X"); plt.ylabel("Y")
plt.title("Clustering Example", fontsize=20); plt.grid(); plt.show()

Figure 5. Sample output from ClusteringExample.py using 1000 random data points and a k-means algorithm with 7 total clusters.

Once the above code has been run, it should produce a figure like the one below. Notice that the clusters intersect each other and some points appear to lay in more than one cluster. This is simply an artifact of our choice to draw the cluster circles based on the most distant point from the cluster mean. Also, our input data was inherently random so one should not expect to see dense clusters.